NIST SQUID MULTIPLEXING SYSTEM SOFTWARE HANDBOOK

Rev 1/31/2003

CDR

Summary

| Program (.py) | ## Use | ## Icon |
| --- | --- | --- |
| essential |  |  |
| muxclock | clock control w/adj LSYNC | muxclock |
| rack_baseline_mux | DFB control single channel | baselinecontrol |
| dfbmuxgui | DFB control in MUX mode | muxcontrol |
| rack_addrboard | ADDR card control | adresscontrol |
| pci_mux_control | configurable PCI card control | pcicontrol |
| squid2.py | tower card control | towercontrol |
| miscellaneous & subprograms |  |  |
| addr_board_dfb | same as rack_addrboard |  |
| dfbmuxgui2 |  |  |
| mux |  |  |
| mux_dfb |  |  |
| mux_rack |  |  |
| mux_reset |  |  |
| rack |  |  |
| clockcard | clock control w/o LYNSC |  |
| squidarraypreamp |  |  |
| pci_writex_mux | stream data to disk |  |
| pci_writex_mux_multiple | under development for 2D mux data streaming |  |
| pci_writex_mux_test |  |  |
| squid3.py |  |  |
| p3631.py | tower power supply config/toggle |  |

# DFB system programs

# Digital feed back board control/configuration

Program	rack_baseline_mux.py

Created

Path

Interface	RS232 to optical

Description	Use this .gui for 1 channel DFB.

Notes	line period=16 by definition, not adjustable, set MUXCLOCK line period 	to 31 for proper operation (any other value causes errors in data 	stream)
data rate =50 Mbit/sec, error/feedback transmitted alternately on LSYNC 	edges
ADC offset should negative and equal 4x[lock voltage in ADC units]

Program	dfbmuxgui.py

Created	GCH 3/02

Path

Interface	RS232 to optical

Description	Use this .gui for multiplexing. 
Controls global variables for multiplexing; number to mux (NMUX, number of rows), number of samples (NSAMP, samples averaged for during line sampling event), settling time (SETT, number of mstr clock cycles to settle before sampling commences), triangle wave out parameters (offset time/offset length/step, use for open loop diagnostics, 1/14/1=1V full scale out). Send local (row specific) variables; ADC offset (lockpoint), DAC offset, P & I gain values (used in feedback algorithm), lock (feedback open/closed), triangle on/off.

Notes	SETT + NSAMP<=LSYNC for proper operation
ADC offset should be positive equal NSAMPx[lock voltage in ADC units] up to NSAMP=16 and constant 16x[lock voltage in ADC units] for NSAMP>16 (truncation kicks in at 16 bits)

Program	rack_addrboard.py

Created	SWN 3/02

Path

Interface	RS232 to optical

Description	Use this .gui to set ADDR board parameters; card address, row address, adclo (off state voltage), adchi (on state voltage), width (on duration in #master clock cycles), delay (delay between LSYNC edge and bias on state)

Notes	When multiplexing, use [1] .gui for each row with a unique row address (and unique card address).
DELAY+WIDTH<=LSYNC

Program	rack.py

Class		DFB

Path

# PCI card control/configuration

Program	pci_mux_control.py

Class		DFB

Path

Description	Use to stream error/feedback data to disk with .gui interface. Allows selection of input channels, number of memory buffers (size of data stream), and a programmable delay in units of master clock cycles for synchronizing DFB cards with PCI (default value 8).

Interface	PCI bus

Notes	to view data use LINUX command 
od –t x4 filename|less(more)

[octal dump hex 4bytes/line]

		need frame bit to trigger stream (won’t work with MUX BASELINE)

Program	pci_writex.py

Class		DFB

Path

Description	Use to stream error/feedback data to disk.

Interface	PCI bus

Syntax		pci_writex.py #(PCI card addr, 0) #(PCI channel, 0-15)  filename

Notes	to view data use LINUX command 
od –t x4 filename|less(more)

[octal dump hex 4bytes/line]

Program	pci_writex_mux.py

Class		DFB

Path

Description	Use to stream error/feedback data in multiplexed mode to disk (Adds frame bit for rowcount=0 for demultiplexing in software).

Interface	PCI bus

Syntax		pci_writex_mux.py #(PCI card addr, 0) #(PCI channel, 0-15)  filename

Notes	to view data use LINUX command 
od –t x4 filename|less(more)

[octal dump hex 4bytes/line]

CLOCK card control/configuration

Program	muxclock.py

Created

Path

Interface	RS232 to optical

Description	Used to set LSYNC (line period) for DFB/ADDR, and to start/stop line clock (useful for resetting system).

Notes	For use with DFB_MUX_BASELINE, line period must be set to 31 for correct data streaming.

Program	clockcard.py

Created

Path	WINDMILL home/pcuser/dfb

Interface	RS232 to optical

Description	Used to start/stop master clock (useful for resetting system).

Notes	Cannot be used for multiplexing (w/ dfbmuxgui.py).

# Miscellaneous

Program	dfb_unpack.c

Class		DFB

Path

Description	unpacks multiplexed data (demux)??

Interface	none

Format		dfb_unpack.py filename

Misc		is this a MATLAB program?

# Tower electronics programs

Program	SQUID2.py

Created

Path	/user/local/src/python/tower

Interface	RS232 to optical

Description	.gui to set bias levels for tower cards

Format	channel = 8 * card addr + card channel
level in DAC units (14 bit, fs 2.5 V?)

Program	P3631.py

Created

Path	/home/pcuser/cardtests???

Interface	GPIB

Description	.gui to configure/toggle Agilent E3631A power supplies for analog (tower) cards

# Test & verification programs

Program	PCIintf.py

Created

Path	/home/pcuser/cardtests

Interface	PCI

Description	PCI test program???

Program	digitalFB_test.py

Created	??

Path	/user/local/src/python/dfb??

Interface	RS232 to optical

Description	.gui for functional testing of DFB

Notes	can get rough measurement of bit noise by changing dip switch settings and monitoring DFB out

# Miscellaneous programs

Program	pyvbox.py

Path	?

Interface	RS232 to optical

Description	set a voltage level of a battery powered ‘blue box’