Program:

# Initialize LS370, E3647A, 34970A

While loop until stop is pushed

While loop for # of sensors to read for i=1 to 12

Read temp from LS370 Chi

Check temp, if <1K, change LS 370 bias voltage to 200 V, if > 1K change LS370 bias to 2.00 mV

Temperature into array size of # channels

End while

If auto-cycle=True AND (if LS370ch2 < 3.5K OR Start Cycle=True) then:

Set Start cycle = True

If (LS370Ch5<3.5 AND LS370Ch3>40) OR 4He In=True Then

4He Charc in=False

If 3He in=True OR LS370Ch5<0.75 Then

E3647Aout1 =0V

E3647Aout2=0V

34970AMod1out4=10V

34970AMod1out5=10V

3He Charc in=False

Else

E3647AOut1=0V

34970AMod1out4=10V

If 3He Charc in=True OR LS370Ch6<20 Then

E3647Aout2=10V

Else

E3647Aout2=0V

Endif

IF LS370Ch6>35 Then

3He charc in=F

Else

3He Charc in=Value of Query (3He Charc in OR Value of “If LS370Ch6<20”)

EndIF

EndIf

Else

If LS370Ch3<40 OR 4He Charc in=True then

E3647Aout1=30V

Else

E3647Aout1=0V

EndIF

IF LS370Ch3>50 Then

4He Charc in=False

Else

4HE Charc in=4He Charc in OR value of  query “if LS370Ch 3<40 K”

ENDIF

If 3He Charc in=True OR LS370Ch6<20 Then

E3647Aout2=10V

Else

E3647Aout2=0V

ENDif

IF LS370Ch6>35K Then

3He charc in=False

Else

3He Charc in=3He Charc in OR Value of query if “	LS370Ch6<20K”

ENDIF

EndIF

4He in=Value of query of (IF LS370Ch5<3.5 AND LS370Ch3>40) OR Value of 4HE in

3He in=Value of query of (If LS370Ch3<.75 AND LS370Ch6>20) OR Value of 3He in

Else

Set Start Cycle, 4He in, 3He in, 4He Charc in, 3He Charc in =False

End IF

Query Voltage outputs from E3647Aout1 and E3647Aout2, calculate powers on Heaters = (Voltage output)^2 * 420 / (480^2)

Query Voltage outputs from 34970A (module 1, channels 4 and 5), calculate powers on HS heaters = (voltage output)^2 / (50k)

Write Temp Data, Power Data, Status of Recycling to defined path

End While

Close LS370,E3647a, 34970A

End program