## Appendix: Configuration of YGOR Managers

In this appendix, we describe, for each YGOR manager, the meta-parameters, or configuration keyword values that are used, and how they relate to the YGOR parameters.  Unless we have made an oversight, any parameter not listed here should retain its M&C default value.

### Managers: when to ignore:

YGOR Managers that are out of service will be ignored and the configuration process will issue a warning message and go ahead and set up as many of the other managers as it can. If a system that constantly monitors the health of all device is not available, then a manager will be assumed to be out of service if its state is 'off', or if it does not respond within a few seconds. Managers whose state is 'standby' will be turned on.

Managers

- A.1 ScanCoordinator

- A.2 SwitchingSignalSelector

- A.3 LO1

- A.4 IFRack

- A.5 ConverterRack

- A.6 ActiveSurface

- A.7 AnalogFilterRack

- A.8 DCR

- A.9 Spectrometer

- A.10 SpectralProcessor

- A.11 BCPM

- A.12. Receivers: PrimeFocus,   Rcvr1_2,   Rcvr2_3,   Rcvr4_6,   Rcvr8_10,   Rcvr12_18,   Rcvr18_26,   Rcvr40_52

### A.1. ScanCoordinator

There is some importance in the order in which parameters are set in what managers. First set 'subsystemSelect', so that the devices will inherit subsequent parameters. When setting the switching parameters, first set the number of phases, then the blanking, and then the others.

As is currently implemented, the switching tables, number phases, and related parameters will be passed from the Scan Coordinator to all selected back ends, and whatever other device also needs it, such as the LO1 manager.  Regardless of whether the DCR is a selected device, the DCR will have its phase table set up identical to that in the Scan Coordinator.

Likewise, the receiver parameter will continue to be passed on to the IFRack and the LO1 Coordinator.

When finished setting all parameters, do a 'regChange', not 'activate', 'start', or 'prepare'. For all other managers, do a 'prepare'

| Table A.1.1. Scan Coordinator |  |
| --- | --- |
| YGOR Parameters | Depends on Keyword(s) |
| receiver | Receiver |
| subsystemSelect | Receiver, Backend |
| switching_signals_master | Backend |
| number_phases | Swmode |
| phase_start | Swmode |
| sig_ref_state | Swmode, Backend, Vframe |
| cal_state | Swmode |
| blanking | Swmode, Swtype, Backend |
| switch_period | Swper |

receiver

Set to keyword value for Receiver. The receiver parameter is passed to other managers that need to know it, namely LO1, and IFRack.

subsystemSelect

- Always include the Antenna, Receiver, LO1, IFRack, IFManager, Switching Signal Selector, Active Surface, and Measurements.

- Include the converter rack, unless the only backend is DCR_IF or DCR_PF.

- Include the backend manager if there is one.

- If the backend is the Spectrometer or DCR_AF, include the analog filter rack.

switching_signals_master 

Set this to the backend, with a few exceptions:

| Table A.1.2. |  |
| --- | --- |
| Backend | switching_signals_master |
| DCR | DCR |
| SpectralProcessor | SpectralProcessor |
| Spectrometer | Spectrometer |
| BCPM | DCR |
| GBPP | DCR |
| BCPM&SP | SpectralProcessor |
| Radar | DCR |
| VLBI | VLBA_DAR |

Note that the Scan Coordinator automatically sends the switching signal master to the Switching Signal Selector.

number_phases, phase_start, sig_ref_state, cal_state 

These parameters all depend on the Switching Mode, as listed in the table.

| Table 4.1.3. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Swmode | number_phases | phase_start | sig_ref_state | cal_state |
| tp (not Spectral Processor) | 2 | [0.0, 0.5] | ['Sig', 'Sig'] | ['NoNoise', 'Noise'] |
| tp (Spectral Processor) | 2 | [0.0, 0.5] | ['Sig', 'Ref'] | ['NoNoise', 'Noise'] |
| tp_nocal | 1 | [0.0] | ['Sig'] | ['NoNoise'] |
| sp | 4 | [0.0, 0.25, 0.5, 0.75] | ['Sig', 'Sig', 'Ref', 'Ref'] | ['NoNoise', 'Noise', 'NoNoise', 'Noise'] |
| sp_nocal | 2 | [0.0, 0.5] | ['Sig', 'Ref'] | ['NoNoise', 'NoNoise'] |

blanking

- If Swmode is 'tp_nocal', blanking = 0.0

- If Backend is VLBI or Obstype is Pulsar, blanking = 0.0

- If Swtype is 'fsw' and Backend is SpectralProcessor, blanking = 0.04

- If Vframe is not Local, and Backend is SpectralProcessor, blanking = 0.04

- For all other cases, blanking = 0.002

switch_period

Set to Swper keyword value.

### A.2. SwitchingSignalSelector

| YGOR Parameters | Keyword(s) |
| --- | --- |
| disableLOBlanking | Backend, Vframe, Swtype |
| disableLocalBlanking | Backend |

disableLOBlanking

Always enable LO Blanking

disableLocalBlanking

Enable Local Blanking for all backends except VLBI.

### A.3. LO1

The LO1 manager controls two synthesizers, LO1A and LO1B, and can route signals from either synthesizer to any of the recievers. For standard configurations, we will always use LO1A as the tracking LO, and LO1B as the test tone. The configurator will always disable the test tone. To use the test tone, the user must use a tweaking command.

Several parameters are always set the same way; for these there is nothing in the "Keyword" column.

| Table A.3.1. LO1 Parameters |  |  |
| --- | --- | --- |
| YGOR Parameters | Keyword(s) | Comments |
| loConfig | -- | Set to 'TrackA_BNotUsed' |
| restFrequency | Restfreq |  |
| ifCenterFreq | Restfreq, Deltafreq, Nwin | See Section 6.0 |
| switchDeltas | Swfreq, Swtype | If Swtype is not 'fsw' set to zeros. |
| sourceVelocity | (Vlow+Vhigh)/2 |  |
| restFrame | Vframe |  |
| velocityDefinition | Vdef |  |
| tolerance | Backend, Nchannels |  |
| phaseCalCtl | Obstype, Phasecal | VLBI only |
| phaseCalMode | Obstype, Phasecal | VLBI only |
| subsystemSelect | ----- | set to [1,1,0,1] |
| useOffsets | ----- | set to 'false' |
| autoSetLOPowerLevel | ----- | set to 1 |
| testToneFreq | ----- | set to 17000.0 |
| testTonePowerLevel | ----- | set to -110 db |

#### restFrequency

restFrequency is set to the first item in the Restfreq list.

#### ifCenterFreq

Set the 'ifCenterFreq' to the value of IF1 as explained in Section 6.0.

#### switchDeltas

Set the switchDeltas to the value of keyword Swfreq. If not frequency switching set them to zero. The other switching table parameters (number_phases, phase_start, sig_ref_state, switch_period, switching_signals_master, blanking, cal_state) are set by the Scan Coordinator so they do not have to be explicitly set by the Configurator.

#### restFrame

Vframe keywords may be abbreviated for convenience, but must be translated to the parameter values required by the LO1 manager, as listed in the following table.

| Table A.3.2. restFrame values |  |
| --- | --- |
| Vframe | LO1 restFrame |
| topo | Local |
| bary | Barycentric |
| lsrk | KinematicalLSR |
| lsrd | DynamicalLSR |
| galac | Galactocentric |
| cmb | CosmicBackground |

#### velocityDefinition

Likewise, the parameter values accepted by LO1 for velocityDefinition are as follows.

| Table A.3.3. velocityDefinition values |  |
| --- | --- |
| Vdef | LO1 velocityDefinition |
| rad | Radio |
| opt | Optical |
| rel | Relativistic |

#### tolerance

The frequency tolerance should be set to 1% of the spectral channel spacing used by the back end.

#### phaseCalCtl and phaseCalMode

- If the Obstype is not VLBI, then phaseCalCtl is set to 'Off'

- If the Obstype is VLBI and Phasecal keyword is 'M1' or 'M5', then phaseCalCtl is set to 'on', and phaseCalMode is set to 'M1' or 'M5'.

- Otherwise, phaseCalCtl should be 'off'.

- If phaseCalCtl is 'on', then the switches that route the phase cal signal to the receiver must also be set.

### A.4. IFRack

The IF Rack receives the IF signals from all the receivers after the first down conversion. Eight switches, each with eight positions, select signals from the receivers. The signals pass through IF filters, and on to optical drivers. Each signal can go to one of two optical drivers, depending on how the transfer switches are set (S9 .. S12).

| Table A.4.1. IF Rack Parameters |  |  |
| --- | --- | --- |
| YGOR Parameters | Keyword(s) | Comments |
| analog_power_level | Receiver, Bandwidth |  |
| filter_select | Bandwidth, Nwin, etc. |  |
| laser_auto_level_control |  | Set all to 'swOn' |
| subsystemSelect |  | Set all to ones. |
| noise_bandwidth |  | Set to 'narrowband' |
| S9 ... S12 |  | Transfer switches: See Section 7.0 |

#### analog_power_level

The 'balance' operation adjusts attenuators to bring the power of each signal to the level specified in the 'analog_power_level' parameter. The configurator does not do a balance in the initial setup, but it does set 'analog_power_level'. The appropriate analog power levels depend on the IF center and bandwidth. The engineering department will provide a table of recommended power levels for IF center and bandwidth.

#### filter_select

- For Prime Focus receivers, filter_select is always 'pass_all', and the IF filter is set by the receiver manager.

- For Gregorian receivers, the IF filters are set to the smallest value that includes "BWTOT", as explained in Section 6. But if BWTOT is greater than 1280 MHz, set the filter to 1280.

- Set the filters for all the optical drivers that are to be used.

#### S9 ... S12 (transfer switches)

These are set according to the results of the path optimization that was discussed in Section 7.

### A.5. ConverterRack

The Converter Rack receives signals from the optical receivers. The signal from each receiver is split to 4 converter modules. There are a total of 16 modules, organized into two racks, known as "Rack A", in which resides modules numbered 1-8, and "Rack B", which has modules numbered 9-16. Each module can select between one of two fibers as its input, using the 'CMInput' parameter.

Eight frequency synthesizers (G1- G8) provide a second down conversion frequency (LO2) to two converter modules each. 'Gfrequency' is the list of these frequencies.

| Table A.5.1. Converter Rack Parameters |  |  |
| --- | --- | --- |
| YGOR Parameters | Keyword(s) | Comments |
| CMInput | Receiver, Beam |  |
| ABselect | Receiver, Beam | Ignored unless backend is the Spectral Processor |
| CMOutput | Backend, Bandwidth |  |
| Gfrequency | Receiver, Backend, Bandwidth, Nwin, Restfreq, Dfreq, Vlow, Vhigh, Vdef | Refer to Section 6 |
| Glevel | --- | Set them all to 10 |
| CMAttenuator | Backend, IF Center |  |

#### CMInput

CMInput is an array of size 16, giving for each of the 16 converter modules "A" or "B" indicating which of two fiber receivers it can select. The cabling file will need to be consulted to determine which CMInputs are from which optical receivers.

#### ABselect

ABselect is used only for the Spectral Processor. Set this to "A" to connect the 8 SP inputs to CM1-8; "B" for CM9-16.

#### CMOutput

CMOutput is a list of 16 numbers indicating filter and output switch settings for each of the 16 converter modules. The switch settings are listed in the table.

| Table A.5.3. |  |  |
| --- | --- | --- |
| CMOutput[n] | Filter | Destination |
| 1 | All Pass | Analog Filter Rack : Bandwidth = 200 or 800 MHz |
| 2 | 500-1000 MHz | VLBI or Radar backends |
| 3 | All Pass | unused |
| 4 | 550 MHz Low-pass | Analog Filter Rack - Bandwidth = 12.5 or 50 MHz Spectral Processor BCPM |

Note that certain backends (VLBI, BCPM, Radar) connect to only a few of the converter modules. One must consult the cabling file to find out which modules connect to which backends. For example, the current cable file specifies:

- VLBI: CM1&5, CM9&13

- Radar: CM10&14

- BCPM1: CM4&8, CM12&16

- BCPM2: CM3&7, CM11&15

#### Gfrequency

Gfrequency is a list of LO2 frequencies as explained in Section 6. The path finding process (Section 7) determines the assignment of synthesizer to spectral window.

#### CMAttenuators

The engineers need to provide a list of suggested values that depend upon the backend and IF Center frequency.

### A.6. ActiveSurface

| YGOR Parameters | Keyword(s) |
| --- | --- |
| correctionSelect | Receiver |

For receivers for frequencies 8 GHz and higher (i.e., X-band, Ku-band, K-band, Q-band, W-band), the active surface FEM model should be enabled. For lower frequency receivers (prime focus, L-, S-, and C- bands), the FEM model should be turned off and the Zero Points mode should be enabled.

#### correctionSelect

- For receivers of frequencies < 8 GHz, correctionSelect = [zero=1, fem=0, random=0].

- For receivers of frequencies > 8 GHz, correctionSelect = [zero=1, fem=1, random=0]

### A.7. AnalogFilterRack

The Analog filter rack receives signals from the Converter Rack, passes them through filters, and sends them to samplers in the Spectrometer or DCR.

There are two flavors of filter module:

- "Sampler Filters" handle the wide-band signals (200 or 800 MHz)

- "Converter Filters" handle the narrow-band signals (12.5 or 50 MHz)

There are eight Sampler Filter modules. They get their inputs from the "all pass" outputs of the ConverterRack. The 'SGInput' parameter tells these modules to select between ConverterRack modules in either rack A or B. The 'SGFilter' parameter selects the bandpass filter: use 'wide' to select 800 MHz bandwidth, and 'narrow' to select 200 MHz.

The outputs of Sampler Filter modules 1 through 8 go to high speed samplers 1 through 8 in the Spectrometer.

There are 16 "Converter Filter" modules. Their inputs are connected to the "550MHz Lowpass" outputs of the Converter Rack modules. Converter filter module numbers 1-16 connect to corresponding numbered converter rack modules. Parameter 'CFFilter' sets the bandpass in these modules. Set CFFilter to 'wide' for 50 MHz and 'narrow' for 12.5 MHz.

Although the possibility exists of setting different bandwidths in different modules, we will not do this for this version of the configurator. All modules that will be used will be set the same.

| Table A.7.1. AnalogFilterRack parameters |  |  |
| --- | --- | --- |
| YGOR Parameters | Keyword(s) | Comments |
| SGInput | Receiver, Beam | SGInput = 1(input from CR "A") or 2(input from CR "B" |
| SGFilter | Bandwidth | SGFilter = 'wide' for 800 MHz, or 'narrow' for 200 MHz |
| CFFilter | Bandwidth | CFFilter = 'wide' for 50 MHz, or 'narrow' for 12.5MHz |
| subsystemSelect |  | Set them all to 1 |

#### "Sampler Filter" (wideband) Connections

Input switch selection and output connections to the Spectrometer and DCR. The cabling file must be searched to find the actual connections.   The current connections are:

| Table A.7.2. Wideband filter module connections |  |  |
| --- | --- | --- |
| SGInput= 1 / 2 | SF Module | Spectrometer Sampler Connection |
| CM 1 / 9 | 1 | J1 |
| CM 2 / 10 | 2 | J2 |
| CM 3 / 11 | 3 | J3 |
| CM 4 / 12 | 4 | J4 |
| CM 5 / 13 | 5 | J5 |
| CM 6 / 14 | 6 | J6 |
| CM 7 / 15 | 7 | J7 |
| CM 8 / 16 | 8 | J8 |

#### "Converter Filter" Connections

The outputs of the Converter Filter modules are hardwired to the low-speed samplers in the Spectrometer. The current connections are listed in the following table but, again, the cabling file will need to be consulted. Samplers are identified by connection numbers J9 to J40, sometimes also designated as low-speed samplers number 0 through 31.

| Table A.7.3. Narrow filter module connections |  |  |  |
| --- | --- | --- | --- |
| ConverterRack module | AFR "Converter Filter Module | Spectrometer Sampler Connection | Sampler number |
| 1 | 1 | J9 | 0 |
| 2 | 2 | J10 | 1 |
| 3 | 3 | J17 | 8 |
| 4 | 4 | J18 | 9 |
| 5 | 5 | J13 | 4 |
| 6 | 6 | J14 | 5 |
| 7 | 7 | J21 | 12 |
| 8 | 8 | J22 | 13 |
| 9 | 9 | J25 | 16 |
| 10 | 10 | J26 | 17 |
| 11 | 11 | J33 | 24 |
| 12 | 12 | J34 | 25 |
| 13 | 13 | J29 | 20 |
| 14 | 14 | J30 | 21 |
| 15 | 15 | J37 | 28 |
| 16 | 16 | J38 | 29 |

(Yes there is actually a good reason for this strangeness!)

### A.8. DCR

The Digital Continuum Receiver records detected power from one of two banks of inputs, bank A and bank B. Sixteen channels of continuum data can be recorded from the selected bank.

Data comes to the DCR from three different devices, the IF Rack, the Prime Focus Receiver, and the Analog Filter Rack. For the purpose of configuring, we are considering only the IF Rack and Analog Filter Rack inputs, and are treating these cases as two different back ends, called "DCR_IF" and "DCR_AF".

The parameters that the configurator needs to be concerned with are:

| Table A.8.1. DCR Parameters |  |
| --- | --- |
| YGOR Parameters | Keyword(s) |
| Bank | Receiver, Beam |
| Channel | Receiver, Beam |
| CyclesPerIntegration | Tint, Backend |

Switching phase table information will be sent from the scan coordinator, so there is no need to set explicitly 'switching_signals_master', 'switch_period', 'number_phases', 'phase_start', 'cal_state', 'sig_ref_state', or 'blanking'.

#### Bank and Channel

Naturally the cabling file must be consulted to find the connections from any receiver/beam to DCR channel. The nominal connections are summarized in Table A.8.2.

| Table A.8.2 DCR Inputs |  |  |
| --- | --- | --- |
| Device | Bank | Channels |
| IF Rack | A | 1 - 8 |
| AFR SG1 - SG8 (wide band channels) | A | 9 - 16 |
| AFR CF1,3,5,7,9,11,13,15 (narrow band channels) | B | 9 - 16 |

#### CyclesPerIntegration

- If the DCR is the switching signal master, then:  CyclesPerIntegration = Tint/Swper

- If the DCR is not the switching signal master, then it is being used to monitor Tsys, so just set CyclesPerIntegration to 1.0/Swper so its integration time is one second.

### A.9. Spectrometer

The Auto-Correlation spectrometer can produce high-resolution spectra with a choice of 4 bandwidths, in a vast variety of modes. The configurator will set up in a limited set of modes. Cross-polarization and pulsar modes will not be considered.

Although it is possible to set up different banks with different bandwidths and modes, we will not consider that. All spectra will have the same bandwidth and number of channels.

| Table A.9.1. Spectrometer Parameters |  |
| --- | --- |
| YGOR Parameters | Keyword(s) |
| configuration | Bandwidth, Nwin, Beam, Nchannels |
| relative_bandwidth | Bandwidth |
| slow_samplers | Nwin, Beam |
| fast_samplers | Nwin, Beam |
| number_slow_samplers | Nwin, Beam |
| slow_samplers_level | Nlevels |
| requested_integration_time | Tint |
| polarization | Obstype |

As is the case for all backends, the switching phase tables parameters are not directly set by the configurator, but by the Scan Coordinator.

#### configuration

The 'configuration' is a string specifying the number of banks and how many quadrants per bank. Of the vast number of possible configurations, we can accomodate all observing setups with six of them. as named in the following table.

| Table A.9.2. ACS Configurations |  |  |
| --- | --- | --- |
| Configuration | Num.Banks | Num.Quads per bank |
| Spectrum_A1 | 1 | 1 |
| Spectrum_A2 | 1 | 2 |
| Spectrum_A4 | 1 | 4 |
| Spectrum_A1_B1 | 2 | 1 |
| Spectrum_A2_B2 | 2 | 2 |
| Spectrum_A1_B1_C1_D1 | 4 | 1 |

#### relative_bandwidth

relative_bandwidth is an array whose length is the number of banks. 
If the Bandwidth is 12.5 MHz or 200 MHz, set this parameter to 'narrow'. Otherwise, set it to 'wide'. Set all banks that will be used to the same value.

#### slow_samplers

The slow samplers are assigned to banks in groups of 8. slow_samplers is an array of size four corresponding to the four groups of samplers: 0-7, 8-15, 16-23, 24-32. (If one refers to Table A.7.3, one notes that only 4 out of each of these groups of 8 are ever used.) For each group, the parameter is set to "Bank_A", "Bank_B", "Bank_C", or "Bank_D' to assign the group of samplers to a bank. It may also be set to "not_used", if a group of samplers is not assigned to any bank.  Only one group of samplers may be assigned to a bank.

#### fast_samplers

fast_samplers is an array of length 8, corresponding to the eight fast samplers. For each sampler, the parameter is set to "Bank_A", "Bank_B", "Bank_C", or "Bank_D" to assign the sampler to a bank. If the sampler is not assigned to a bank, the parameter is set to "not_used".

For Bandwidth=200MHz, no more than 4 fast samplers may be assigned to a bank. 
For Bandwidth=800MHz, no more than 2 fast samplers may be assigned to a bank.

#### number_slow_samplers

number_slow_samplers is an array whose length is the number of banks. 
Set them if using slow samplers, i.e., the Bandwidth is 12.5 or 50 MHz. 
The TOTAL number of samplers is the number of spectral windows (Nwin) times the number of beams (Nbeams) times two (we always set up for dual polarization. Divide the total number of samplers by the number of banks to get the number of samplers per bank. Set all banks to this number.

#### slow_samplers_level

slow_samplers_level is an array whose length is the number of banks. 
Set them all to Nlevels (i.e., 3 or 9).

#### requested_integration_time

requested_integration_time is an array whose length is the number of banks. 
Set them all to Tint.

#### polarization

polarization is an array whose length is the number of banks. 
Set them all to zero for non-cross-polarization mode.

### How to choose the configuration

The appropriate configuration and assignment of samplers to banks depends on the Bandwidth, the number of spectral windows (Nwin), number of beams, and number of channels. We set up always for dual polarization.

The allowed combinations of Nwin and number of beams for the slow samplers (of which we can use up to 16 at a time), are given in Table A.9.3.

| Table A.9.3. Total number of samplers for Low-speed modes. |  |  |  |
| --- | --- | --- | --- |
| Nwin | one beam | two beams | four beams |
| 1 | 2 | 4 | 8 |
| 2 | 4 | 8 | 16 |
| 4 | 8 | 16 |  |
| 8 | 16 |  |  |

The next table, A.9.4, shows a typical set of configuration choices to use for each choice of Nwin and beam for the slow samplers.  The cabling file will describe the actual routing between a beam and the backend.  In the table, beam designations B1, B2, B3, and B4 apply not only to multi-beam receivers, but also to any receiver at all. Each "beam" indicates a pair of inputs to the IF Rack, as follows:

- B1 : IF Rack inputs S1,S3; (route thru conv. Rack A)

- B2 : IF Rack inputs S5,S7; (route thru conv. Rack B)

- B3 : IF Rack inputs S2,S4; (route thru conv. Rack A)

- B4 : IF Rack inputs S6,S8; (route thru conv. Rack B)

Currently, eight-IF (Nwin=8) mode requires the receiver IFs to be split to both converter racks A and B.  Thus the receiver output is going to fibers 1,3,5,7 for beam B12, or to fibers 2,4,6,8 for beam B34.   Sampler groups are designated as: G1 = (0-7); G2 = (8-15); G3 = (16-23); G4 = (24-31)

| Table A.9.4. Low-Speed Configuration and sampler assignment. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nwin | Beam | Total num. samplers | Configuration | Levels | num.samplers per bank | Sampler Group | Nchannels |
| 1 | B1 | 2 | A1 A2 A4 A1 A2 A4 | 3-lev 3-lev 3-lev 9-lev 9-lev 9-lev | 2 | G1 | 32768 65536 131072 8192 16384 32768 |
| 1 | B2 | 2 | same as for B1 |  | 2 | G3 |  |
| 1 | B3 | 2 | same as for B1 |  | 2 | G2 |  |
| 1 | B4 | 2 | same as for B1 |  | 2 | G4 |  |
| 1 | B12 | 4 | A1_B1 A2_B2 A1_B1 A2_B2 | 3-lev 3-lev 9-lev 9-lev | 2 | G1_G3 | 32768 65536 8192 16384 |
| 1 | B34 | 4 | same as for B12 |  | 2 | G2_G4 |  |
| 1 | B1234 | 8 | A1_B1_C1_D1 A1_B1_C1_D1 | 3-lev 9-lev | 2 | G1_G2_G3_G4 | 32768 8192 |
| 2 | B1 | 4 | A1 A2 A4 A1 A2 A4 | 3-lev 3-lev 3-lev 9-lev 9-lev 9-lev | 4 | G1 | 16384 32768 65536 4096 8192 16384 |
| 2 | B2 | 4 | Same as 2 IFs, B1 |  | 4 | G3 |  |
| 2 | B3 | 4 | Same as 2 IFs, B1 |  | 4 | G2 |  |
| 2 | B4 | 4 | Same as 2 IFs, B1 |  | 4 | G4 |  |
| 2 | B12 | 8 | A1_B1 A2_B2 A1_B1 A2_B2 | 3-lev 3-lev 9-lev 9-lev | 4 | G1_G3 | 16384 32768 4096 8192 |
| 2 | B34 | 8 | Same as 2 IFs, B12 |  | 4 | G2_G4 |  |
| 2 | B1234 | 16 | A1_B1_C1_D1 A1_B1_C1_D1 | 3-lev 9-lev | 4 | G1_G2_G3_G4 | 16384 4096 |
| 4 | B1 | 8 | A1_B1 A2_B2 A1_B1 A2_B2 | 3-lev 3-lev 9-lev 9-lev | 4 | G1_G2 | 16384 32768 4096 8192 |
| 4 | B2 | 8 | same as 4 IFs, B1 |  | 4 | G3_G4 |  |
| 4 | B3 | 8 | same as 4 IFs, B1 |  | 4 | G1_G2 |  |
| 4 | B4 | 8 | same as 4 IFs, B1 |  | 4 | G3_G4 |  |
| 4 | B12 | 16 | A1_B1_C1_D1 A1_B1_C1_D1 | 3-lev 9-lev | 4 4 | G1_G2_G3_G4 G1_G2_G3_G4 | 16384 4096 |
| 4 | B34 | 16 | A1_B1_C1_D1 A1_B1_C1_D1 | 3-lev 9-lev | 4 4 | G1_G2_G3_G4 G1_G2_G3_G4 | 16384 4096 |
| 8 | B1 or B2 | 16 | A1_B1_C1_D1 A1_B1_C1_D1 | 3-lev 9-lev | 4 4 | G1_G2_G3_G4 G1_G2_G3_G4 | 16384 4096 |
| 8 | B3 or B4 | 16 | A1_B1_C1_D1 A1_B1_C1_D1 | 3-lev 9-lev | 4 4 | G1_G2_G3_G4 G1_G2_G3_G4 | 16384 4096 |

Now for the fast samplers, i.e., Bandwidths of 200 or 800 MHz, Table A.9.5 shows the possible combinations of Nwin and number of beams. There are a maximum of 8 fast samplers.

| Table A.9.5. Total Numbers of Fast Samplers. |  |  |  |
| --- | --- | --- | --- |
| Nwin | one beam | two beams | four beams |
| 1 | 2 | 4 | 8 |
| 2 | 4 | 8 |  |
| 4 | 8 |  |  |

A typical set of possible fast sampler configurations are listed in Table A.9.6.  The actual set depends upon the cabling file.

| Table A.9.6. Configurations for Fast Samplers. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Nwin | # beams | NSamp | Configuration | Bandwidth | Sampler assignment | NChannels |
| 1 | 1 | 2 | A1 A2 A4 A1 A1_B1 A2_B2 | 200 200 200 800 800 800 | 01 01 01 01 0_1 0_1 | 8192 16384 32768 2048 4096 8192 |
| 1 | 2 | 4 | A1 A1_B1 A2_B2 A1_B1 A1_B1_C1_D1 | 200 200 200 800 800 | 0123 01_23 01_23 01_23 0_1_2_3 | 4096 8192 16384 2048 4096 |
| 1 | 4 | 8 | A1_B1 A1_B1_C1_D1 A1_B1_C1_D1 | 200 200 800 | 0123_4567 01_23_45_67 01_23_45_67 | 4096 8192 2048 |
| 2 | 1 | 4 | A1 A1_B1 A2_B2 A1_B1 A1_B1_C1_D1 | 200 200 200 800 800 | 0123 01_23 01_23 01_23 0_1_2_3 | 4096 8192 16384 2048 4096 |
| 2 | 2 | 8 | A1_B1 A1_B1_C1_D1 A1_B1_C1_D1 | 200 200 800 | 0123_4567 01_23_45_67 01_23_45_67 | 4096 8192 2048 |
| 4 | 1 | 8 | A1_B1 A1_B1_C1_D1 A1_B1_C1_D1 | 200 200 800 | 0123_4567 01_23_45_67 01_23_45_67 | 4096 8192 2048 |

Note: the sampler assignment column shows which samplers are assigned to which banks, for example, '01' means samplers 0 and 1 are assigned to bank A; '01_23' means samplers 0 and 1 are assigned to bank A and samplers 2 and 3 are assigned to bank B.

### A.10. SpectralProcessor

The Spectral Processor is a general purpose Fourier-transform spectrometer which may be used either for normal spectroscopy or for pulsar timing. It will do autocorrelations of its input signals, and will also do cross-correlations of pairs of inputs (usually cross-polarization pairs).

The first table lists the YGOR parameters that the configurator needs to set. Following the table are descriptions of how each YGOR parameter relates to the configuration keywords. Finally, there is an explanation of how to connect the Spectral Processor to the GBT IF system.

| A.10.1. Parameters set directly by configurator |  |
| --- | --- |
| YGOR Parameters | Dependent on Keyword(s) |
| processorMode | Obstype |
| multiplierMode | spmode |
| reqBandwidth | Bandwidth |
| reqNumChan | Nchannels, Bandwidth, Nwin, Beam |
| numSpectr | Nwin, Beam |
| observeFreq | Nwin, Beam, Restfreq, Dfreq |
| rfSideband | Receiver |
| reqIF | (normally these are always 250 MHz |
| sampleTime | Tint |
| calPhase | always set to 0.0 |
| calDuration | always set to 0.5 |
| atodLevMode | set to 'Immediate' |
| balance | set to 'NotBalance' |
| reqPulsarPeriod | set to Swper |
| reqDispersionMeasure | set to zero |
| polycoDatFile | optional user input in pulsar mode. |

The switching phase table and related information is set by the Scan Coordinator, and does not need to be set explicitly by the configurator. These parameters include: number_phases, cal_state, sig_ref_state, phase_start, blanking, switch_period, and switching_signals_master.

processorMode

- For Obstype = 'Spectroscopy', processorMode = 'StdSpecLine'

- For Obstype = 'Pulsar', processorMode = 'SyncFreqTime'

multiplierMode 

This is a parameter selected by the user. The values are 'Square', 'Cross', or 'SqrCross'. 'Square' means parallel polarization products; 'Cross' means crossed polarization products; 'SqrCross' means both. See the table below for how this relates to other parameters.

reqBandwidth

- For Bandwidth = 40 MHz, reqBandwidth = '_40MHz'

- For Bandwidth = 20 MHz, reqBandwidth = '_20MHz'

- etc.

reqNumChan and numSpectr 
The possible values for these parameters are listed in the following table:

| Table A.10.2. Spectral Processor Configurations |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bandwidth | multiplierMode | NIF | reqNumChan | numSpectr |
| 40 MHz | Square or Cross | 1 | 1024 | 1 |
| 20 MHz | Square or Cross or SqrCross | 1 | 1024 | 1 |
| 20 MHz | Square or Cross | 2 | 512 | 2 |
| 10 MHz | Square, Cross, or SqrCross | 1 | 1024 | 1 |
| 10 MHz | Square, Cross, or SqrCross | 2 | 512 | 2 |
| 10 MHz | Square or Cross | 4 | 256 | 4 |
| <= 5 MHz | Square, Cross, or SqrCross | 1 | 1024 | 1 |
| <= 5 MHz | Square, Cross, or SqrCross | 2 | 512 | 2 |
| <= 5 MHz | Square, Cross, or SqrCross | 4 | 256 | 4 |

Note that NIF = Nwin x Number of beams.

observeFreq

observeFreq is an array of NIF frequencies. Calculation of these frequencies is described in Section 6. Use "Flocal[i]" from Section 6.0 for spectral window i.

rfSideband

rfSideband is an array of size NIF containing the sideband designation (either 'Upper' or 'Lower') for each IF. These are the net sideband, and depend on the receiver:

- 'Lower' for Rcvr8_10 and lower frequencies.

- 'Upper' for Rcvr12_18 and higher frequencies.

reqIF

reqIF is an array of size NIF. For standard configurations (which is all that we are considering), all members are set to 250 MHz.

atodLevMode and balance

The configurator should always set these to 'Immediate' and 'NoBalance' for the initial setup.

### SpectralProcessor connections

Here we make an attempt to explain the logic of connecting the SpectralProcessor to the GBT IF system.

The Spectral Processor has 8 inputs, designated A1,A2,A3,A4, and B1,B2,B3,B4. The "A" inputs connect to one polarization, usually X or LCP, the "B" inputs to the other polarization. For normal dual-polarization observing, A1 and B1 are used for the first spectral window, A2 and B2 for the second, and so on. There is no flexibility in these assignments, i.e., one cannot use A2 and B2 for the first window. If using one spectral window one must always use A1 and B1; if using two windows, one must always use A1,A2, B1,B2; and so on.

The Spectral Processor inputs connect to an "A/B" switch to the outputs of the converter modules. The switch in the "A" position connects the 8 SP inputs to outputs of converter modules 1-8, and the "B" position to modules 9-16. Do not confuse the "A/B" switch with the "A" and "B" of the Spectral Processor inputs!!

Given a particular instance of the cabling file, there are only two ways to connect the Spectral Processor, one through Converter Rack A ("A/B" switch in "A" position), the other through Converter Rack B.

The following table describes how the GBT IF system connects to the Spectral Processor. It shows the typical use of converter modules, but keep in mind that the cabling file must be consulted to determine how the modules are actually cabled up. Polarization pairs (X,Y) or (LCP,RCP) are normally routed through fiber pairs (1,3), (2,4), (5,7), or (6,8), which can be selected by converter module pairs CM1&5, CM2&6, CM3&7, CM4&8, CM9&13, CM10&14, CM11&15, or CM12&16. The actual fiber pairs must be determined by consulting the cabling file in case one or more fiber drivers are out of service.

The output switch of the converter module in all cases is set to 'LPF550MHz'.

| Table A.10.3. Spectral Processor Inputs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fibers | Polarization | Converter Modules | A/B Switch position | Spectral Processor Input |
| 1 or 2 | XL | CM1 | A | A1 |
| 3 or 4 | YR | CM5 | A | B1 |
| 1 or 2 | XL | CM2 | A | A2 |
| 3 or 4 | YR | CM6 | A | B2 |
| 1 or 2 | XL | CM3 | A | A3 |
| 3 or 4 | YR | CM7 | A | B3 |
| 1 or 2 | XL | CM4 | A | A4 |
| 3 or 4 | YR | CM8 | A | B4 |
| 5 or 6 | XL | CM9 | B | A1 |
| 7 or 8 | YR | CM13 | B | B1 |
| 5 or 6 | XL | CM10 | B | A2 |
| 7 or 8 | YR | CM14 | B | B2 |
| 5 or 6 | XL | CM11 | B | A3 |
| 7 or 8 | YR | CM15 | B | B3 |
| 5 or 6 | XL | CM12 | B | A4 |
| 7 or 8 | YR | CM16 | B | B4 |

For the single-beam receivers (Rcvr8_10 and lower frequencies), signals are routed to both converter racks. If there is a bad module in converter rack A (CM1-8), then an alternate route is to use rack B.

For multi-beam receivers, typically each beam is routed through either rack A or B, but not both. Beams usually work in pairs that can be toggled if beam switching. One member of the pair goes through converter rack A (selected by A/B switch in A position), the other through rack B (selected by A/B switch in B position). Since the Spectral Processor can connect to either rack A or B, but not both, it cannot record both beams of a pair simultaneously.

### A.11. BCPM

The BCPM is a pulsar data acquisition machine designed and built at the universities in Berkely and CalTech. It can do pulsar timing observations (operating_mode='timing'), and can also write out the raw high-speed samples for later pulsar searches (operating_mode='search'). Before any data taking, one must set it up in 'monitor' mode to set its power levels.

There are two BCPMs, designated BCPM1 and BCPM2. Each can accept an input signal with a bandwidth of 192 MHz. With one BCPM, one spectral window can be observed. With two BCPMs, two spectral windows can be observed. The "normal" situation will be that spectral window #1 goes to BCPM1 and spectral window #2 goes to BCPM2. Since at the moment, only BCPM1 is working only one spectral window is currently possible.

A complication is that BCPM2 will have the ability to collect cross-polarization data and BCPM1 does not. Some observers will want to use only BCPM2.

The first six YGOR parameters in the table are really the only configuration parameters, specified by user keywords. The remaining ones may change from one observation to the next. The table gives reasonable values to use for the initial setup.

| Table A.11.1. BCPM Parameters |  |
| --- | --- |
| YGOR Parameters | Keyword(s) |
| submanagers_used | Nwin |
| setif | Receiver |
| center_frequency | Restfreq, Deltafreq |
| channel_bandwidth | (user selected) |
| sample_time | (user selected) |
| sum_polarizations | (user selected) |
| cal_used_flag | (initialize to 'no') |
| operating_mode | (initially set to 'monitor') |
| data_storage | (initially set to 'disk' |
| file_size | (initialize to 60) |
| base_name | (source name) |
| target_name | (source name) |

submanagers_used

- Use the user-set keyword from Secion 3

- If Nwin=2 and both BCPMs are working, set to 'bcpm1_and_bcpm2'

setif

- setif=1 to select the BCPM "A" inputs, A1 and A2, which connect to modules in converter rack A.

- setif=2 to select the BCPM "B" inputs, B1 and B2, which connect to modules in converter rack B.

- setif=3 to receive inputs from telescope 85-3. So if the path that the configurator has chosen for the selected receiver leads to converter rack A, setif=1, otherwise setif=2.

center_frequency

This is an array of two frequencies, the first for BCPM1, the second for BCPM2. These are the two sky frequencies, Flocal[i], at described in Section 6.0.

### BCPM Connections

Each BCPM has two inputs, one for each polarization. Each BCPM is connected to a pair of converter modules. The converter module output switch is set to 'LPF550MHZ'. A typical setup is described in the following table. But of course the cabling file must be consulted to find which modules actually connect to the BCPMs.

| Table A.11.2. BCPM Inputs |  |  |
| --- | --- | --- |
| Polarization | Converter Module | BCPM: Input |
| XL | 3 | BCPM2 : A1 |
| YR | 7 | BCPM2 : A2 |
| XL | 4 | BCPM1 : A1 |
| YR | 8 | BCPM1 : A2 |
| XL | 11 | BCPM2 : B1 |
| YR | 15 | BCPM2 : B2 |
| XL | 12 | BCPM1 : B1 |
| YR | 16 | BCPM1 : B2 |

### A.12 Receivers

Most of the receivers use the same parameter names.  We will list the common parameters then discuss each receiver in turn.  In the table, ‘xx’ will be different from one receiver to another.  Note that one has to be aware that the consistency of parameter names between receivers is not very good.  We are assuming that ‘tuning_frequency’ will be implemented for all receivers so that polarization hybrids and LNA bias settings can be set by the manager and not the configuration tool..

Table A.12.1 Parameters for most receivers

| #### Parameter | #### Depends upon Keywords | #### Notes |
| --- | --- | --- |
| xxBiasSwitch |  | Always on |
| xxExtToMCBCtrlSel |  | Always ext |
| cpuLoCalPwrSw |  | Always on |
| xferSwCtlMode | SwType | Ext if polarization switching, otherwise MCB |
| xx IfFilterSwitch or ifChannelXFilterBank | Bandwidth, Nwin, etc. | In many cases, the filter is an RF, not an IF filter. |
| loOrHiCalSel | Noisecal |  |
| polarizationSelect | Polarization |  |
| xferSwitch | Polswitch | Ignored by the manager if xferSwCtlMode is in ext |
| Tuning_frequqncy | Restfreq, Velmin, velmax, etc. | Not implemented on some receivers |
| IfChannelXAttenuator |  | Only available on the PF receivers. Set to value determined by the engineer |
| Notch_filter | Notchfilter | Currently available on Rcvr1_2. |

Table A.12.2 Parameters for Rcvr12_18, Rcvr18_26, and Rcvr40_52

| #### Parameter | #### Depends upon Keywords | #### Notes |
| --- | --- | --- |
| xxBiasSwitch |  | Always on |
| calState |  | Always ext |
| noiseSourcePwrSwX |  | Always on |
| xferState | SwType | Ext if beam switching, otherwise MCB |
| xx IfFilterSwitch | Bandwidth, Nwin, etc. | Most receivers do not have a set of filters |
| XferSwX or xxXferSw | Beamswitch | Ignored by the manager if xferState is in ext |
| ChopperCtrl |  | Always off; available currently available only on Rcvr40_52. |
| Tuning_frequqncy | Restfreq, Velmin, etc. | Not yet implemented |

xxBiasSwitch

Turns on the LNA amplifiers

xxExtToMCBCtrlSel  or calState

Specifies that the noise diode should be controlled by the switching signal master

NoiseSourcePwrSwX or cpuLoCalPwrSw

Turns on the noise diode

xferState or xferSwCtlMode

Specifies that the polarization or beam switch should be in external control for either polarization or beam switching.  Some receivers do not have transfer switches.

XferSwX, xxXferSw or xferSwitch

If not beam or polarization switching, specifies the position of the transfer switch.  Some receivers do not have transfer switches.  Ignored if the xferState or xferSwCtlMode are set to external.

notch_filter

Whether the notch filter is to be used.  Only the RCvr1_2 currently has such a filter.

loOrHiCalSel

Whether high or low noise diode are to be used.

polarizationSelect

Whether the observation will use linear or circular polarizations.  Not all receivers have this capability.

xxIfFilterSwitch or ifChannelXFilterBank

The name implies these are I.F. filters but in fact the name is used for either IF or RF filters.  Whether it is an I.F. or R.F. filter depends upon the receiver.  The filter should encompass the full range of sky frequencies or I.F. as described in section 6.

Tuning_frequqncy

As described in section 6, this should be the frequency at the middle of the observer’s observing frequency range.  All receivers need this parameter so that the manager can automatically set polarization hybrids settings, attenuators, or LNA biases.  Unfortunately, only a few receivers have this capability.

IfChannelXAttenuator

Attenuators currently are only on the PF receivers.  The receiver engineer should provide the attenuator values and the criteria by which they are set

chopperCtrl

Currently chopperCtrl is only available on the 40-52 GHz receiver.  The chopper should be ‘retracted’ or off.